# James Sturgess - 20475519

# MMME3085 Software Project – Robot Writer

# Software Description

The software communicates with a drawing robot to draw out some text at a user defined height. It does this by first asking the user for a text height in mm, it asks the user until a value in the range is give. The software calculates the scale factor to later scale the font data using, scale factor = input/18. The software opens and reads the SingleStrokeFont.txt file and stores this data, all x and y values in this data for each glyph are then scaled using the scale factor calculated earlier. The test.txt file is then read and stored as a string  all on one line. The characters in this string are then compared with the scaled font data and a new array is made that contains the relevant font data in an array that corresponds to this string. All lines in this array containing the 999, character codes and point count numbers are removed to result in an array with purely scaled x, y and penState values. This is then formatted into gcode and the com port opened. The variable containing all gcode lines is sent line by line until it is all sent and the com port is closed.

# Project Files

The project uses the following files

- main.c – the main script developed from the skeleton provided

- rs232.c – contains the functions to use the com port

- rs232.h – The header file for rs232.c

- serial.c – contains the functions to replicate the serial port

- serial.h – The header file for serial.c

- SingleStrokeFont.txt – Text file containing all the required font data to draw out the text

- test.txt – Text file containing the text or sentence to be drawn out by the drawing robot

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| textHeightUnscaled | float | Needs to store user input as a decimal |
| strokePoint | struct | Needs to store 3 data points for each pen stroke in array |

# Functions

float getTextHeight();

returns the unscaled user input text height (in mm)

int loadFontData(const char *fontDataFile, struct fontData *f);

opens and reads the fontdata file and stores all information in a struct

Parameters:

fontDataFile – pointer to read the file

f – pointer to store the data in the fontData struct

void scaleFontData(struct fontData *f, float scaleFactor);

scales the stored font data by multiplying all x and y values by the scale factor

Parameters:

f – pointer to fontData to read x and y values

sclaeFactor – the scale factor by which the x and y values need to be multiplied by

int loadTextFile(const char *testFile, char *buffer, int maxLength);

opens and reads the test.txt file, concatenates and stores all of this data

Parameters:

testFile – pointer to read the text file

buffer – pointer to store the read text

maxLength – the max number of characters the can be stored

const struct glyph* searchForGlyphData(const struct fontData *font, int charCode);

compares the character codes between the text.txt and the stored font data

Parameters:

font – pointer to font data to find all corresponding character codes

charCode – the character code for each character in test text to look up to compare with

int convertTextToGlyhs(const struct fontData *font, const char *text, struct glyph *textGlyphs, int maxGlyphs);

takes test.txt and makes a new array with corresponding glyph data in one place for that sentence.

Parameters:

font – pointer to find font data to map characters to glyphs that need to be drawn out

text – pointer to store glyph data in new array

textGlyphs – pointer to new array where data is to be stored

maxGlyphs – max number of glyphs new array can hold

void shiftGlyphArray(struct glyph *textGlyphs, int gCount, float shift);

shifts each consecutive glyph by shifting the x values and then when the x values exceed 100 it moves to the next lines down.

Parameters:

textGlyphs – pointer to glyph array

gCount – number of glyphs in array

shift – the value of the shift that each glyph needs to be shifted by

int makeRawGlyphCoordData(const struct glyph *textGlyphs, int gCount, float (*rawGlyphs)[3]);

makes new array with only x, y and penState values, removing all 999, char code and point count lines from previous array.

Parameters:

textGlyphs – pointer to glyph array with x and y and penState values

gCount – count for number of glyphs

rawGlpyhs – pointer to output variable with only x, y and penstate values for entire text to be drawn out

int makeGCode(const float (*rawGlyphData)[3], int totalLines, char ***gCodeLines);

converts array into g code format for lines to be sent one by one when robot is ready

Parameters:

rawGlyphData – pointer to array with x, y and penState values

totalLines – number of lines used to as count so knows when to stop

gCodeLines – pointer to output lines of gcode to be stored then sent

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| main | Normal use |  |  |
| getTextHeight | Ask user for input in range | 6 | Return user input for scaling in main |
| loadFontData | Open txt file in read more | Failed to load | Print error code and return 1 |
| scaleFontData | Read stored font data | Load font data and scale factor | Output scaled font data by scale factor |
| loadTextFile | Open txt file in read mode | Successfully open | Output variable containing text in txt |
| convertTextToGlyhs | Load text and fontdata | Successfully loaded font data and text data | Output array of glyph data for string |
| shiftGlyphArray | Shift down | Is x value of stroke greater than 100? | Take away 100 from x value and shift y value down to next line |
| makeGCode | Array containing x, y, penState values |  | Output array containing lines of formatted in gcode stlye |

# AI Statement

AI used in debugging to help understand compiler errors.

# Flowchart(s)

Included as a separate pdf